Tender document (VS; shifting lock)

VERSCHI vario + (VS) for bonded and unbonded base courses

The shifting lock is a metal construction consisting of a high-quality steel alloy with additional hot-dip galvanising and colour protection. Nine threaded holes of 5 mm/8 mm are provided to take up earth spikes.

Steel grade: Special allowance

Corrosion protection via hot-dip galvanising min. 10μ,

Powder coating min. 80μ

Colour: blue with the marking / VERSCHI vario +

Various functional properties are achieved based on the special metal construction. A pressure joint of 4/8 mm is ensured by design. The horizontal support surface absorbs the dead loads of the deposited element. Therefore, the overall weight is fixed on the VERSCHI vario +.

In addition, mounted spikes can penetrate through the foundation into the base course. The load weight of the covering fixes the VERSCHI vario + to its location. Joint gauges (8 mm) should be used.

For bonded base courses, the base course must be slit. Observe separate instructions from the supplier (installation instructions).

The VERSCHI vario + is to be used only in the hazardous areas designated in the planning. When securing a closed crossbar, each format must be secured in a row of the lane, this secured row forms the anchorage in the lane.

The anchoring points can be taken from the construction drawing or the installation plan.

Always ensure a complete, continuous installation.

Number:

Model: VERSCH vario +                                                                        Number of pieces 	...................

Trademarks must be observed

Sampling

A sample area for large-scale projects with the built-in VERSCHI vario + must be created for the client.

A technical sample must be presented to the client for equivalent products. A sample area of min. 1 m² with the selected shifting lock must be created.

For optimal installation, anchoring aids from OPTIMAS (Anker Fix) can be used.

Supplier information:

NUPFAHL GmbH

Müggenburger Str. 24a / 20539 Hamburg

Tel. (040) 780 90  9-50

Fax (040) 780 90  9-25

www.nupfahl.de

Email: info@nupfahl.de